NASA DEVELOP National Program

International Research Institute for Climate and Society (IRI)

Summer 2015

Malawi Disasters II

Applications of Flood Definitions and NASA Earth Observations to Create a Flood Forecasting Methodology

Project Team:

Andrew Kruczkiewicz (Project Lead), andrewkruczkiewiczi@gmail.com

Helen Cen

Brigitte Moneymaker

Advisors & Mentors:

Dr. Pietro Ceccato (Lead Environmental Monitoring Program, International Research Institute for Climate and Society, The Earth Institute, Columbia University)

Past or Other Contributors:

Jerrod Lessel

Alex Sweeney

Partner Organizations

Red Cross/Red Crescent Climate Centre (RCRCCC), Boundary Organization, POC: Erin Coughlan, Senior Climate Specialist

Malawi Red Cross Society, Collaborator and End-User, POC: Hastings Kandaya, Director- Programmes and Development

Applied Sciences National Applications Addressed:

Disasters

Study Area:

Malawi

Study Period:

January 1, 2015 to January 31, 2015

Earth Observations & Parameters

Aqua and Terra, MODIS - Water body delineation (DFO flood product)

Aqua and Terra, LANCE MODIS - Flood detection (NRT-GFM flood product)

TerraSAR-X, Synthetic Aperture Radar (SAR) - Flood detection

RADARSAT, Synthetic Aperture Radar (SAR) - Flood detection

RADARSAT-2, Synthetic Aperture Radar (SAR) - Flood detection

TRMM, Precipitation Radar (PR) - Rainfall

TRMM, TRMM Microwave Imager (TMI) - Rainfall

TRMM, Visible and Infrared Scanner (VIRS) - Rainfall

Aqua, Advanced Microwave Scanning Radiometer Earth Observing System (AMSR-E) - Rainfall

Defense Meteorological Satellite Program (DMSP) satellites, Special Sensor Microwave Imager (SSM-I) – Meteorology data

Aqua, Microwave precipitation Estimates (AMSU-A) – Rainfall

ESA SMOS (Soil Moisture and Ocean Salinity), Microwave Imaging Radiometer using Aperture Synthesis (MIRAS) radiometer – Soil Moisture

EUMETSAT METOP, Advanced Scatterometer (ASCAT) Soil Water Index

Models Utilized

University of Maryland/University of Washington Dominant river tracing-Routing Integrated with VIC Environment (DRIVE) model

Software Utilized

ArcGIS - Raster manipulation/analysis, map creation of Dartmouth Flood Observatory, NASA NRT-GFM Flood Mapping Products, TerraSAR-X, RADARSAT, RADARSAT-2, and ground truth data

80-100 Word Objectives Overview

During extreme climate events, such as the January 2015 Malawi flood disaster, humanitarian organizations rely on Earth observation (EO) data to evaluate impact and design response programs. This project builds on the previous analysis of various EO flood detection products and incorporates specific flood definitions to better identify impacts of flash floods versus riverine floods. These definitions will then be incorporated into a methodology to better monitor and forecast flash flood events in vulnerable areas.

Abstract

In January 2015, Malawi, in southern Africa, experienced a series of flood events, which resulted in the displacement of over 230,000 residents and left 276 people reported dead or missing. During the disaster, the combination of extreme rainfall on a short time scale, but large spatial area resulted in both river-overflow floods and flash floods. Differences between these distinct flood types affected communities in Malawi differently, and the resulting relief effort was hampered by insufficient information provided to responders. From a partnership with the Malawi Red Cross and supported by a review of media reports, it was shown that there were challenges in order to reach the affected population in a timely manner. A previous comparative analysis of several spatial inundation products by the Malawi Disasters Spring 2015 team concluded the following; 1. There is inconsistency in the spatial distribution of a flood signal across flood products relative to a specific flood type and 2. Some flood products seem to be useful in detecting flash floods while others, seem useful for detecting riverine floods. This project aims to produce a  methodology for analyzing flood detection skill relative to flash floods to inform a framework for better monitoring and forecasting those events in vulnerable areas. Along with previous flood products and NASA Earth Observing systems, this project will incorporate several ESA (European Space Agency) Satellites to incorporate soil moisture as an important variable in flash flood detection. It is expected that the results of this study will increase the ability to monitor different types of flood events, which will benefit organizations involved with disaster relief efforts in Malawi; allowing for a quicker response and more appropriate allocation of emergency flood relief efforts.

Community Concerns

- January 2015 floods in Malawi resulted in 276 deaths, 153 people missing, and over 230,000 people displaced. The affected population is in urgent need of nutritional supplies, shelter, and drinkable water.

- Due to the spatial and temporal characteristics of flash floods, flood detection products are rarely able to identify them. Communities affected by the flash floods may be overlooked by disaster responders, or delay relief aid.

- Potential to improve flash flood detection and prediction using satellite products is not only vital for local preparation, but also for enhancing the efficiency of relief aid delivery.

Current Management Practices & Policies

Remote regions in Africa often have sparse meteorological and ecological satellite and ground data. Project partners in this region rely heavily on remotely sensed data as it increases the temporal and geospatial scales of projects. Data from NASA satellites and sensors (e.g. MODIS onboard Aqua and Terra and TRMM) enable project partners working in these regions to better evaluate the impact extent and develop response programs.

Decision Support Tools

| End-Product | Earth Observations Used | Benefit & Impact |
| --- | --- | --- |
| Flash Flood Monitoring Methodology | SMOS, EUMETSAT METOP, TRIMM, MODIS, Aqua, Terra, TerraSAR-X, RADARSAT, RADARSAT-2 | Malawi Red Cross and other decision makers will be better equipped to develop more effective response programming. |

Benefit to End-User:

- Creation of a comprehensive methodology to accurately monitor and predict flash flood events in Malawi to enable end-users to better monitor spatial extent of different flood types and improve response programs.

- The methodology could potentially inform the development of a flash flood early warning system in Malawi.