NASA DEVELOP National Program

2018 Spring Project Proposal

Virginia – Wise

Fremont River Basin Water Resources

Assessment of Available Water from Annual Snow Pack in the Fremont River Basin Based on NASA Earth Observations and In Situ Data

Project Overview

Project Synopsis: The Fremont River in Utah provides year-round irrigation for the agricultural lands of Rabbit Valley lying in the upper Fremont River basin. The upper Fremont River basin contains a plethora of surface cover types and landforms consisting of volcanic rocks, high mountain valleys, wet meadows, cold fisheries, and irrigated agriculture lands being influenced from snowpack melt. This river also runs through the Capitol Reef National Park (NP), which depends on the river for supplying needed water. The water supply in the river is highly influenced by the annual snow pack. There are currently 16,000 acres within the watershed that are irrigated for agricultural purposes. The primary use of land in the watershed is livestock production, including the growing of alfalfa and grass hay. This project will employ MODIS data to map and monitor snow pack for 2000-present. Suomi National Polar-orbiting Partnership (Suomi-NPP) Visible Infrared Imaging Radiometer Suite (VIIRS) data will also be used to generate snow maps. Snow maps from the project will be fed into the Snowmelt-Runoff Model (WinSRM) to efficiently predict snow melt. These snow melt predictions will then be compared to USGS stream gage data to determine the amount of water being input into the Fremont River basin.

Community Concern: The river flow forecasts based on snowpack data and the observed Fremont River flows through Capitol Reef National Park do not agree, but sufficient data are lacking to explain these differences. The river flows have diminished despite several years of higher than average snowpack in the Dirty Devil River watershed. Consequently, reductions in irrigation water from the Fremont River are being requested by the Utah Division of Water Resources. The requests have become more common in recent years, although the acreage under irrigation has remained constant. Capitol Reef NP manages historic orchards and pastures in the Dirty Devil River watershed that rely on irrigation water from the Fremont River. Currently, the National Resources Conservation Service (NRCS) creates forecasts for the Dirty Devil River downstream of Capitol Reef, but it does not partition out flows for the Fremont River. The objective of this project is to produce snow mapping products that could be used with modeling to help assess stream flow trends in the Fremont River basin.

Source of Project Idea: This idea originated from the project partners Rebecca Weissinger of the Northern Colorado Plateau Network, and Terry Fisk, Chief of Resource Management and Science, at Capitol Reef National Park.

National Application Areas Addressed: Water Resources, Agriculture & Food Security

Study Location: Fremont River Basin, UT

Study Period: 2000 – 2017 (September – March)

Advisors: Joseph Spruce (Consultant to Science Systems & Applications, Inc.), Dr. L. DeWayne Cecil (NOAA National Center for Environmental Information, Global Science & Technology, Inc.), Bob VanGundy (The University of Virginia’s College at Wise)

Partner Overview

Partner Organizations:

| Organization | POC (Name, Position/Title) | Partner Type | Boundary Org? |
| --- | --- | --- | --- |
| National Park Service, Northern Colorado Plateau Network | Rebecca Weissinger, Ecologist | End User | No |
| National Park Service, Capitol Reef National Park | Terry Fisk, Chief of Resource Management and Science | End User | No |

End-User Overview

End User’s Current Decision-Making Process:

Currently, remote sensing is not incorporated into the decision-making process in water allotment for the National Park Service within the Fremont River basin. The existing management practices are provided via state and federal agencies involved in water management. Irrigation reduction is currently the only decision made to help conserve water. The Utah Division of Water Resources has directed reduced water use, even when snowpack is above the normal average providing more usable water than usual.

End User’s Capacity to Use NASA Earth Observations:

National Park Service, Northern Colorado Plateau Network – The National Park Service is familiar with NASA Earth observations as they incorporate Landsat and MODIS vegetation indices. This project would introduce the partners to the use of remote sensing for monitoring and assessing water resources, expanding the role NASA Earth observations in the National Park Service’s water management activities.

National Park Service, Capitol Reef National Park – The National Park Service uses NASA Earth observations for vegetation indices in daily operations. This project would introduce the partners to the use of remote sensing for monitoring of snow cover, expanding the role NASA Earth observations in the National Park Service’s daily operations.

Project Communication & Transition Overview

In-Term Communication Plan: The team will communicate project issues and progress with Terry Fisk and Rebecca Weissinger of the National Park Service via bi-weekly teleconferences or as deemed necessary or feasible. The main DEVELOP POC will be the project lead at the Virginia – Wise node.

Transition Plan: Hand off will be conducted either via teleconference with shared screen or Google Hangout. The tools produced by this project will be used by the partners to identify areas where snow melt is impacting the Dirty Devil River and Capitol Reef National Park.

Earth Observations Overview

Earth Observations:

| Platform & Sensor | Parameter(s) | Use |
| --- | --- | --- |
| SRTM | Digital Elevation Model | SRTM elevation values will be incorporated into models for water resources. |
| Terra MODIS | Snow Indices; Total Column Water Vapor | Incorporation of MODIS data will allow for a more accurate spatio-temporal representation of snowpack, water loss due to natural processes, and for tracking sublimation from snowpack. |
| TRMM/GPM | Rainfall Averages | TRMM and GPM data will be used to monitor rainfall averages for the use in The Snowmelt-Runoff Model (WinSRM). |
| Aqua MODIS | Snow Indices | Incorporation of MODIS data will allow for a more accurate representation of snowpack. |
| Suomi NPP VIIRS | Snow Indices | This will be used to compare VIIRS snow maps to those from Aqua and Terra MODIS NDSI. |

Ancillary Datasets:

USGS National Hydrography Datasets (NHD) – defining the hydrologic units for the Watersheds

USGS National Hydrography Datasets Plus (NHDPlus) – defining the hydrologic units for the Watersheds at a high resolution

USGS Watershed Boundary Dataset (WBD) – defining the areal extent of surface water drainage to a point

USGS stream gauge data – investigation of relationships between snow and watersheds

USDA Snowpack Telemetry (SNOTEL) Snow Water Equivalent (SWE) – measurements of snow depth, precipitation, and temperature for use in forecasting water supplies

USDA ForWarn system includes some MODIS based products that show areas predominantly covered by snow. Such products are refreshed every 8 days.

Modeling:

Soil and Water Assessment Tool (SWAT) (Dr. Kenton Ross, NASA Langley Research Center)

Snowmelt-Runoff Model (WinSRM) (Dr. Kenton Ross, NASA Langley Research Center)

North American Land Data Assimilation Systems (Dr. Kenton Ross, NASA Langley Research Center)

Software & Scripting:

Google Earth Engine – Data processing and image classification

MATLAB - Data Processing, statistical analysis, and graphing

Esri ArcGIS – Data processing and analysis

ERDAS Imagine – Classification of Landsat imagery

R – Data Processing, statistical analysis, and graphing

QGIS – Data visualization, processing, and analysis

Decision Support Tool & End Product Overview

End Products:

| End Products | Partner Use | Datasets & Analyses | Software Release Category |
| --- | --- | --- | --- |
| Fremont River Snow Melt Map | This map will be incorporated into the decision-making processes of the partners to help them plan current and future irrigation practices. | The water map will use Aqua and Terra MODIS snow indices along with in situ river gauge data to monitor the snow melt input into the Fremont River from The Snowmelt-Runoff Model. | II |
| Snow Cover Time Series | This time series will be used with the Fremont River Forecasting Map for analysis and planning of future irrigation use. | This time series will date back to 2000 using Aqua and Terra MODIS indices. | II |

End-User Benefit: A down-scaled river forecasting model for the Fremont River at the USGS gauge at Bicknell (09330000) and a time series of snow extent in the Fremont River watershed upstream of the gauge will enhance current methods of forecasting water resources throughout the Dirty Devil River watershed. Better planning for irrigation water use and better anticipation of calls for voluntary reductions in irrigation are a some of the decisions that may be impacted by the results of this project.

Project Timeline & Previous Related Work

Project Timeline: 2 Terms: 2018 Spring to 2018 Summer

Multi-Term Objectives:

- Term 1(Proposed Term): 2018 Spring (VA) – Fremont Water Resources

  - The first term will investigate temperature and precipitation measurements from MODIS, VIIRS, and GPM, and their effects on stream flows in the Fremont River. The project will also generate Terra and Aqua MODIS snow maps (from NDSI and Saito S3 snow indices) that will help monitor snow pack and snow melt during the snow season. Maps of snowpack extent and location at 8-day intervals will be used for comparison with USGS Watershed Boundary Dataset (WBD). The seasonal river volume will also be determined by incorporating in situ river gauge data. The first term will also include preparations for the subsequent term that include getting the WinSRM model up and running.

- Term 2: 2018 Summer (VA) – Fremont Water Resources II

  - This term will build upon the foundation of the first term and expand the scope of the project to estimate water diversions for irrigations upstream of the park. This will include use of the WinSRM or comparable model to provide estimates of how future irrigation demands will change with air temperature increase if agricultural land use remains static.

Related DEVELOP Work:

Spring 2017 (LaRC) – Glacier National Park Climate II: A Threshold-Based Decision Tree Approach to Mapping Landscape Disturbance in Glacier National Park

Spring 2014 (LaRC) – Chile Water Resources: Using NASA Earth Observation Data to Understand Snowmelt and Address Ongoing Drought in Central Northern Chile

Spring 2017 (MCHD) – Southeastern Arizona Water Resources: Using NASA Earth Observations to Assist the National Park Service in Assessing Snow Cover Distribution and Persistence Changes in the Sky Islands

Summer 2016 (WC) – Northern Great Plains Water Resources: Discovering Archaeological Sites by Utilizing NASA Earth Observations to Detect Changes in Snowpack Coverage in Intermountain National Parks

Fall 2016 (WC) – Northern Great Plains Water Resources II: Utilizing NASA Earth Observations to Detect Changes in Annual Snowpack Coverage in Intermountain National Parks

Notes & References:

Notes:

- There are several SNOTEL sites in nearby mountains

- Interested in Google Earth Engine

  - Want a code they can continue to use in the future

- Above average snow packs- June to early July water cuts take place

- Partners have people with R coding experience

References:

Utah Department of Environmental Quality. (2002). Fremont River Watershed Water Quality Management Plan. Retrieved from https://deq.utah.gov/ProgramsServices/programs/water/watersheds/docs/2006/09Sep/FREMONT_WQMP.pdf

National Park Service U.S. Department of the Interior. (2004) Water Resources Management Plan Capitol Reef National Park Utah. Retrieved from https://www.nature.nps.gov/water/planning/management_plans/care_final_screen.pdf

Y. Asaoka and Y. Kominami. (2013). Incorporation of satellite-derived snow-cover area in spatial snowmelt modeling for a large area: determination of gridded degree-day factor. Annals of Glaciology, 54(62), 205-213.

U.S. Department of the Interior, Bureau of Land Management (2005). Appendix 1-Wild and Scenic River Suitability Considerations. Richfield RPM. Retrieved from: https://eplanning.blm.gov/epl-front-office/projects/lup/68293/86878/104135/Richfield_Appendices.pdf